1	Background	1

2	Using this document	1

3	Hierarchical Structure	1

4	File Information	5

RECID 00	File Information Record	5

5	Sampling Location Records	5

RECID 90	Sampling Platform Record	5

RECID 91	Station / Sampling Event Record	6

RECID 92	Station / Site Description Record	7

6	TRANSECT Record	7

RECID 40	Transect Description Record	7

7	Sample Records	9

RECID 03	Contaminant, Biological Effects And Litter Sample Record	9

RECID 34	Biological Community and Litter Sample Record	11

RECID 04	Biota Specimen Data Record	12

8	measurement records	13

RECID 10	Parameter Measurement Record	13

RECID 36	Litter Record	15

RECID 38	Biological Community Abundance/Biomass Record	16

9	Method RecordS	17

RECID 20	Sampling Method Record	17

RECID 21	Analytical Method Record	19

10	Quality Assurance (QA) Records	21

RECID 93	Reference Material Record	21

RECID 94	Intercomparison Record	21

11	SUPPORTING INFORMATION	22

12	CHANGE LOG	24

# Background

ICES has been asked to support the reporting of environmental data in an integrated reporting format ensuring that the various scientific requirements for specific data types are covered while allowing standardised reporting of common meta data for these data. The format and underlying quality control procedures are designed to ensure comparable and relevant data for international assessments. The requirements in this document have been specified by various Expert and Assessment groups.

This format can be used to report data on: contaminants, biological effects of contaminants, and fish disease in biota (data type CF), contaminants and biological effects in sediment (data type CS), and contaminants, biological effects and ocean acidification in seawater (data type CW). It also allows reporting of biological community data for phytoplankton (data type PP), zooplankton (data type ZP), phytobenthos (data type PB) and zoobenthos (data type ZB). The newest record in the reporting format allows the reporting of litter (data type LT). Litter can be reported as an addition to existing data types files (for example microplastics in stomachs or sediment fractions) or as a data type reported separately on its own (for example, trawl litter data).

This format provides a common entrance to the ICES database DOME which utilizes a common top level structure based on position and time. The reporting format is Version 3.2.5 of the ICES Environmental Data Reporting Formats.

# Using this document

Not all records and fields are mandatory for entering data into the DOME database. Only fields marked “M” are mandatory for DOME. However, the data submitter should check Commission guidelines (HELCOM, OSPAR) since additional fields are mandatory for certain Commission monitoring programmes. These are not reflected in this document. Some fields are marked “r” for recommended to bring these to the users’ attention. This document (and the data submissions created through the use of this format) has been designed for use in connection with a number of web-based facilities.

Links to resources:

| https://vocab.ices.dk/ | Reference codes are found here, i.e. valid lookup codes for the reporting format fields |
| --- | --- |
| https://datsu.ices.dk/web/selRep.aspx | Valid fields and checks according to each data type |
| https://vocab.ices.dk/?sortby=Description&ref=90 | List of recommendations from Commissions and Working Groups can be found in “MPROGinfo” (see Monitoring Programme information). This is no longer kept up-to-date |
| https://dome.ices.dk/datsu/datsu.aspx | Data screening programme |
| https://www.ices.dk/data/guidelines-and-policy/Pages/default.aspx | Commissions and ICES TIMES guidelines |
| https://ices.dk/data/tools/Pages/Submission%20status.aspx | Track the progress of a data submission |
| https://www.ices.dk/data/data-portals/Pages/DOME.aspx | DOME portal information |
| https://dome.ices.dk/browse/index.aspx | View and download submitted files |
| https://dome.ices.dk/views/ | Search DOME with specific filters |

# Hierarchical Structure

This format is based on a hierarchical structure which is illustrated in the diagrams below. Note that the diagram does not specify actual hierarchy for QA records.

# File Information

## RECID 00	File Information Record

| File Information Record – mandatory record – |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Field Code | Field Name | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | ‘00’ | M |
| 2 | RLABO | Reporting institute code | cf. RLABO | M |
| 3 | CNTRY | Country code | cf. CNTRY | M |
| 4 | MYEAR | Monitoring year | YYYY | M |
| 5 | RFVER | Reporting format version number | ‘3.2.5’ | M |
| 6 | DCFLG | Data centre flag - Reserved | multiple options possible |  |

# Sampling Location Records

## RECID 90	Sampling Platform Record

| Sampling Platform Record – mandatory record – |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | ‘90’ | M |
| 2 | SHIPC | Platform / Ship code | cf. SHIPC | M |
| 3 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | Any character 0–9, A–Z etc. | M |
| 4 | Owner | Data owner | Any character 0–9, A–Z etc. | R |
| 5 | PRDAT | Public release date | Date YYYYMMDD | R |
| 6 | DCFLG | Data centre flag - Reserved | multiple options possible |  |

## RECID 91	Station / Sampling Event Record

| Station Record – mandatory record – |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | ‘91’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | Any character 0–9, A–Z etc. | M |
| 4 | LATIT | Latitude (degrees/minutes/decimal minutes or as decimal degrees) Report as WGS84 | 90 00.000 to +90 00.000 or -90.0000 to +90.0000 cf. Field Code Descriptions | M |
| 5 | LONGI | Longitude (degrees/minutes/decimal minutes or as decimal minutes) Report as WGS84 | 180 00.000 to +180 00.000 or 180.0000 to +180.0000 cf. Field Code Descriptions | M |
| 6 | POSYS | Positioning system | cf. POSYS multiple options possible (separate multiple entries with “~” (ascii 126)) | R |
| 7 | SDATE | Sampling date | YYYYMMDD | M |
| 8 | STIME | Sampling time/start (UTC) | 0000–2359 (hhmm) | R Fish disease |
| 9 | ETIME | Sampling end time (UTC) | 0000–2359 (hhmm) See field description for handling time when sampling continues to the next day. | R Fish disease |
| 10 | WADEP | Water depth (sounding in meters) | 0–9 | rCW, rZB |
| 11 | STATN | Station name | cf. STATN Any character 0–9, A–Z etc. (max. 50) | R |
| 12 | MPROG | Monitoring programme | cf. MPROG multiple programmes possible (separate multiple entries with “~” (ascii 126)) | M |
| 13 | WLTYP | Water/land types (river basin/eurotypes) | cf. WLTYP | R |
| 14 | MSTAT | Type of monitoring station cf. Eurowaternet Technical report number 97 | cf. MSTAT (replaces point source) (separate multiple entries with “~” (ascii 126)) | R |
| 15 | PURPM | Purpose of monitoring | cf. PURPM (separate multiple entries with “~” (ascii 126)) | R |
| 16 | EDATE | Sampling end date | YYYYMMDD | mPassive samplers |
| 17 | DCFLG | Data centre flag - Reserved | multiple options possible |  |

## RECID 92	Station / Site Description Record

| Station / Site Description Record – optional for CF/CS/CW/PP/PB/ZP/ZB/LT – |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name | Data type |  |  |  |  |  |  |  | Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | CF | CS | CW | PP | PB | ZP | ZB | LT | ‘92’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | CF | CS | CW | PP | PB | ZP | ZB | LT | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | CF | CS | CW | PP | PB | ZP | ZB | LT | Any character 0–9, A–Z etc. | M |
| 4 | RSRVD | Reserved |  |  |  |  |  |  |  |  |  |  |
| 5 | MATRX | Matrix analysed | CF | CS | CW | PP | PB | ZP | ZB | LT | “SI” | M |
| 6 | PARAM | Parameter code | CF | CS | CW | PP | PB | ZP | ZB | LT | cf. PARGROUP parameters, for example, S-DES | M |
| 7 | MUNIT | Measurement unit | CF | CS | CW | PP | PB | ZP | ZB | LT | cf. MUNIT | M |
| 8 | VALUE | Value | CF | CS | CW | PP | PB | ZP | ZB | LT | Any format | M |
| 9 | DCFLG | Data centre flag - Reserved | CF | CS | CW | PP | PB | ZP | ZB | LT | multiple options possible |  |

# TRANSECT Record

## RECID 40	Transect Description Record

For the purposes of this format, a transect for benthos data is defined as a straight line from shore/transect start, along the substrate, until the end of vegetation or 30 meter depth. This record is used to describe the whole transect. Sections should not be defined here.

A diagram is available in 11 SUPPORTING INFORMATION to use as a guide.

| Transect Description Record - mandatory for PB/ZB/LT transects - |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name | Data type |  |  |  |  | Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier |  | PB |  | ZB | LT | ‘40’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) |  | PB |  | ZB | LT | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) |  | PB |  | ZB | LT | Any character 0–9, A–Z etc. | M |
| 4 | TRANS | Transect ID (must be unique for STNNO) |  | PB |  | ZB | LT | Any character 0–9, A–Z etc. | M |
| 5 | TRDGR | Transect direction degrees (compass heading) |  | PB |  | ZB | LT | 0–9 | R |
| 6 | POSYS | Positioning system |  | PB |  | ZB | LT | cf. POSYS multiple options possible (separate multiple entries with “~” (ascii 126)) |  |
| 7 | LATRS | Transect start Latitude (degrees/minutes/decimal minutes or as decimal degrees) Report as WGS84 |  | PB |  | ZB | LT | 90 00.000 to +90 00.000 or -90.0000 to +90.0000 cf. Field Code Descriptions |  |
| 8 | LNTRS | Transect start Longitude (degrees/minutes/decimal minutes or as decimal minutes) Report as WGS84 |  | PB |  | ZB | LT | 180 00.000 to +180 00.000 or 180.0000 to +180.0000 cf. Field Code Descriptions |  |
| 9 | TRSLN | Length of whole transect (m) |  | PB |  | ZB | LT | 0–9 |  |
| 10 | TREDT | Transect end determination |  | PB |  | ZB | LT | cf. TREDT | mPB, mZB |
| 11 | LATRE | Transect end Latitude (degrees/minutes/decimal minutes or as decimal degrees) Report as WGS84 |  | PB |  | ZB | LT | 90 00.000 to +90 00.000 or -90.0000 to +90.0000 cf. Field Code Descriptions |  |
| 12 | LNTRE | Transect end Longitude (degrees/minutes/decimal minutes or as decimal minutes) Report as WGS84 |  | PB |  | ZB | LT | 180 00.000 to +180 00.000 or 180.0000 to +180.0000 cf. Field Code Descriptions |  |
| 13 | DEPAD | Depth adjustment |  | PB |  | ZB |  | cf. DEPAD | M |
| 14 | TREDP | Transect end depth (distance to surface or water depth) (m) |  | PB |  | ZB | LT | 0–9 |  |
| 15 | MXVEG | Maximum depth where vegetation is found (m) |  | PB |  | ZB |  | 0–9 |  |
| 16 | SPVEG | Species found at Depth limit |  | PB |  | ZB |  | cf. WoRMS |  |
| 17 | RLIST | Reference code list used for species ID |  | PB |  | ZB |  | cf. RLIST |  |
| 18 | DCFLG | Data centre flag - Reserved |  | PB |  | ZB | LT | multiple options possible |  |

# Sample Records

## RECID 03	Contaminant and Biological Effects Sample Record

This sample record is used to group data sampled at one location/time as reported in the sampling event record (91). It is up to the submitter to determine how to group the data (i.e., how many sample records there should be). As a guide, first group according to data type (CF, CS, CW), then according to sampler type (SMTYP).

To reduce the number of sample records needed for biota data (CF), a sample record should be considered to be a haul record (although there is no requirement for this). Decide whether to aggregate according to hauls or not (NOAGG). As a minimum, one sample record will be necessary for each species. If the submitter wishes to organize data according to individual hauls, one sample record can be considered a haul record for all individuals of one species. Actual reporting of the individual fish is done via the biota specimen record (04) in the sub-sample field (SUBNO). Further grouping according to sex can be done via the biota specimen record (04) where the number of individuals in the sub-sample is specified (NOINP).

A sample for sediment data is each core or grab. A sub-sample is a slice or a portion of the grab. One sub-sample can have multiple matrices (SEDtot, SED63 etc.). Decide whether to aggregate (NOAGG). “Grouping” data together can be implied if deemed necessary by the submitter. This could occur if a boxcore has been taken and a multi-corer is then used to take cores from the original large sample. The data can be identified by using a sampler type (SMTYP) which states the use of a 2-step sampling process (contact ICES for code). If other samples are taken from the same location at the “same” time, the actual time field (ATIME) can be used for further grouping – or – a new sampling event record (91) can be created.

Litter sample record (DTYPE = LT) in litter files should be reported with Record ID 34 (see below).

(If litter has been collected at the same time as biota, seawater or sediment data, litter can be reported in a type 36 record directly under a type 03 sample record. If microlitter is found in the organism, litter parameters are reported via a type 10 parameter record.) – This option is not implemented in the database, contact ICES for more information.

| Contaminant, biological effects and litter sample record – mandatory record for CF, CS, CW – |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name | Data types |  |  | Valid value | Mandatory/ Recommended/ Optional |  |
| 1 | RECID | Record identifier | CF | CS | CW | ‘03’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | CF | CS | CW | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | CF | CS | CW | Any character 0–9, A–Z etc. | M |
| 4 | DTYPE | Data type | CF | CS | CW | cf. DTYPE | M |
| 5 | SMPNO | Sample identification (for each species in haul, each sediment core, each sediment grab, each water bottle) | CF | CS | CW | Any character 0–9, A–Z etc. | M |
| 6 | SMLNK | Sampling method link | CF | CS | CW | 1–999 | R |
| 7 | ATIME | Actual time of sampling (UTC) | CF | CS | CW | 0000–2359 (hhmm) | r Tidal waters |
| 8 | NOAGG | Number of aggregated samples (hauls, sediment cores or grabs) taken to comprise sample | CF | CS | CW | 2–99 |  |
| 9 | SPECI | Species of specimen | CF |  |  | cf. WoRMS | mCF |
| 10 | RLIST | Reference code list used for species ID | CF |  |  | cf. RLIST | mCF |
| 11 | FINFL | Factors potentially influencing guideline compliance and interpretation of data | CF | CS | CW | cf. FINFL multiple options possible (separate multiple entries with “~” (ascii 126)) | r Fish disease r CF |
| 12 | DCFLG | Data centre flag - Reserved | CF | CS | CW | multiple options possible |  |

## RECID 34	Biological Community and Litter Sample Record

A diagram is available in 11 SUPPORTING INFORMATION to use as a guide for PB/ZB transects.

Litter sample record (DTYPE = LT) in litter files should be reported with Record ID 34.

(If litter has been collected at the same time as biological community data, litter can be reported in a type 36 record directly under a type 34 sample record.) – This option is not implemented in the database, contact ICES for more information.

| Biological community and litter sample record – mandatory record for PP, ZP, PB, LT– |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name | Data types |  |  |  |  | Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | PP | PB | ZP | ZB | LT | ‘34’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | PP | PB | ZP | ZB | LT | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | PP | PB | ZP | ZB | LT | Any character 0–9, A–Z etc. | M |
| 4 | DTYPE | Data type | PP | PB | ZP | ZB | LT | cf. DTYPE | M |
| 5 | TRANS | Transect ID |  | PB |  | ZB | LT | Any character 0–9, A–Z etc. | M if transect data |
| 6 | SMPNO | Sample identification (for each sediment core, each sediment grab, each water bottle, each transect section or point) | PP | PB | ZP | ZB | LT | Any character 0–9, A–Z etc. For transects, sequential numbering from transect start beginning with 1 | M |
| 7 | SMLNK | Sampling method link | PP | PB | ZP | ZB | LT | 1–999 | M if transect data |
| 8 | ATIME | Actual time of sampling (UTC) | PP | PB | ZP | ZB | LT | 0000–2359 (hhmm) |  |
| 9 | NOAGG | Number of subsamples combined if sample is an aggregation | PP | PB | ZP | ZB | LT | 2–99 |  |
| 10 | FNFLA | Fauna-flora found |  | PB |  | ZB |  | ’Y’ for Yes or ‘N’ for No | M if N |
| 11 | FINFL | Factors potentially influencing guideline compliance and interpretation of data | PP | PB | ZP | ZB | LT | cf. FINFL multiple options possible (separate multiple entries with “~” (ascii 126)) | rZB,rLT |
| 12 | SMVOL | Total sampled volume (litre for PP, cubic meter for ZP, and litre sediment for ZB) | PP |  | ZP | ZB | LT | 1–9 | rPlankton |
| 13 | WIRAN | Wire angle degree | PP |  | ZP |  |  | 0–9 |  |
| 14 | CLMET | Calculation method (for FLVOL) |  |  | ZP |  |  | cf. CLMET | m |
| 15 | FLVOL | Calculated flow volume of water (cubic meter) |  |  | ZP |  |  | 0–9 |  |
| 16 | SUBST | Bottom substrate type |  | PB |  | ZB | LT | cf. SUBST | rPB |
| 17 | DEPOS | Sediment deposits on plants and bottom substrate |  | PB |  | ZB |  | cf. DEPOS |  |
| 18 | PCNAP | Percent coverage by non-attached plants |  | PB |  | ZB |  | 1–100 |  |
| 19 | PRSUB | Percent of bottom covered with the particular bottom substrate type |  | PB |  | ZB | LT | 1–100 | rPB, rLT |
| 20 | TRSCS | Start of tansect section (meters from transect start to section start. (TRSCS=TRSCE if transect point is sampled) |  | PB |  | ZB | LT | 0–9 | mPB |
| 21 | TRSCE | End of transect section (meters from transect start to section end TRSCS=TRSCE if transect point is sampled) |  | PB |  | ZB | LT | 0–9 | mPB |
| 22 | NPORT | Number of portions in split sample (SMPNO) | PP |  | ZP |  |  | 1–9999 | rZP |
| 23 | TRCSD | Distance to surface (m) (water depth) at start of transect section (if transect point is sampled, TRCSD=TRCED) |  | PB |  | ZB | LT | 0–9 | mPB |
| 24 | TRCED | Distance to surface (m) (water depth) at end of transect section (if transect point is sampled, TRCSD=TRCED) |  | PB |  | ZB | LT | 0–9 | mPB |
| 25 | DCFLG | Data centre flag - Reserved | PP | PB | ZP | ZB | LT | multiple options possible |  |

## RECID 04	Biota Specimen Data Record

| Biota Specimen Data Record – mandatory record for CF – |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | ‘04’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | Any character 0–9, A–Z etc. | M |
| 4 | SMPNO | Sample identification (for each species in haul, each sediment core, each sediment grab, each water bottle) | Any character 0–9, A–Z etc. | M |
| 5 | SUBNO | Sub-sample identification (each fish, egg, bird or aggregate pool of same species. New species – new sample record and SMPNO) | Any character 0–9, A–Z etc. | M |
| 6 | NOINP | Number of individuals in sub-sample (i.e. 1 individual or number in pool) | 01–99999 | M |
| 7 | ORGSP | Origin of specimen | cf. ORGSP | r Imposex |
| 8 | SEXCO | Sex code | cf. SEXCO | R |
| 9 | STAGE | Stage of development | cf. STAGE | R |
| 10 | CONES | Condition of specimen | cf. CONES | R |
| 11 | ASTSA | Animal state at time of sampling | cf. ASTSA | O |
| 12 | NODIS | Number of diseases looked for during a fish disease survey | 0–99 | r Fish disease |
| 13 | BULKID | Bulk identification (for individuals only) | If an individual (or parts thereof) has been analysed in one or more bulks, insert the SUBNO identification(s) of the bulk(s). Note that BULKID can only refer to a SUBNO within the same sample. See field descriptions for an example. (separate multiple entries with “~” (ascii 126)) | r Fish disease r Biological effects data |
| 14 | DCFLG | Data centre flag - Reserved | multiple options possible |  |

# measurement records

## RECID 10	Parameter Measurement Record

| Parameter Measurement Record – mandatory record for CF, CS, CW – |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name | Data type |  |  |  |  |  |  | Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | CF | CS | CW |  |  |  |  | ‘10’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | CF | CS | CW |  |  |  |  | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | CF | CS | CW |  |  |  |  | Any character 0–9, A–Z etc. | M |
| 4 | SMPNO | Sample identification (for each species in haul, each sediment core, each sediment grab, each water bottle) | CF | CS | CW |  |  |  |  | Any character 0–9, A–Z etc. | M |
| 5 | SUBNO | Sub-sample identification (each fish, egg, bird, aggregate, grab portion or core slice) | CF | CS | CW |  |  |  |  | Any character 0–9, A–Z etc. | mCF mCS |
| 6 | MATRX | Matrix analysed | CF | CS | CW |  |  |  |  | cf. MATRX | M |
| 7 | DEPHU | Upper depth/pressure (m) For pressure, use PRFLG in parameter record Upper depth of core slice for sediment. | CF | CS | CW |  |  |  |  | 0–9 | R, mCW for most parameters |
| 8 | DEPHL | Lower depth/pressure (m) Use PRFLG in parameter record for pressure |  | CS | CW |  |  |  |  | 0–9 | R, mCW for most parameters |
| 9 | PARAM | Parameter code | CF | CS | CW |  |  |  |  | cf. PARAM | M |
| 10 | MUNIT | Measurement | CF | CS | CW |  |  |  |  | cf. MUNIT | M |
| 11 | BASIS | Basis of determination | CF | CS | CW |  |  |  |  | cf. BASIS | R for chemicals |
| 12 | AMLNK | Analytical methods link | CF | CS | CW |  |  |  |  | 1–999 | Depends on parameter |
| 13 | VFLAG | Validity flag | CF | CS | CW |  |  |  |  | cf. VFLAG multiple flags possible (separate multiple entries with “~” (ascii 126)) |  |
| 14 | QFLAG | Qualifier flag | CF | CS | CW |  |  |  |  | cf. QFLAG |  |
| 15 | VALUE | Value | CF | CS | CW |  |  |  |  | Any format | M |
| 16 | PERCR | Percentage recovery - to be applied (if thought necessary by data submitter) to the reported value (in VALUE field) at an assessment to give a better approximation of the real value | CF | CS | CW |  |  |  |  | 1–100 | O |
| 17 | SIGND | Significant digits reported in VALUE | CF | CS | CW |  |  |  |  | 0–9 |  |
| 18 | UNCRT | Uncertainty value | CF | CS | CW |  |  |  |  | 0–9 | M (chemicals) |
| 19 | METCU | Method of calculating uncertainty | CF | CS | CW |  |  |  |  | cf. METCU | M |
| 20 | DETLI | Limit of detection value | CF | CS | CW |  |  |  |  | 0–9 |  |
| 21 | LMQNT | Limit of quantification | CF | CS | CW |  |  |  |  | 0–9 | R |
| 22 | PRFLG | Pressure flag |  |  | CW |  |  |  |  | ’Y’ for Yes or blank |  |
| 23 | DCFLG | Data centre flag - Reserved | CF | CS | CW |  |  |  |  | multiple options possible |  |

## RECID 36	Litter Record

| Litter Record – mandatory record for LT – |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Data type | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | LT | ‘36’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | LT | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | LT | Any character 0–9, A–Z etc. | M |
| 4 | TRANS | Transect ID | LT |  | M if transect data |
| 5 | SMPNO | Sample identification (for each sediment core, each sediment grab, each water bottle, each transect section or point) | LT | Any character 0–9, A–Z etc. For transects, sequential numbering from transect start beginning with 1 | M |
| 6 | SUBNO | Sub-sample identification (each piece of litter) (ID number from the lab, or a running number) | LT | Any character 0–9, A–Z etc. | M |
| 7 | MNDEP | Minimum depth of sample (metre) For litter on seafloor, upper depth of gear | LT | 0–9 |  |
| 8 | MXDEP | Maximum depth of sample (metre) For litter on seafloor, lower depth of gear - will often be sounding depth | LT | 0–9 |  |
| 9 | MATRX | Matrix analysed | LT | cf. MATRX | M |
| 10 | LTREF | Litter reference list | LT | cf. LTREF | M |
| 11 | PARAM | Parameter code | LT | cf. parameters listed in pargroup “LT” or parameters in LTREF lists | M |
| 12 | LTSZC | Litter size | LT | cf. LTSZC |  |
| 13 | MUNIT | Measurement unit | LT | cf. MUNIT | M |
| 14 | QFLAG | Qualifier flag | LT | cf. QFLAG |  |
| 15 | VALUE | Value measured | LT | Any format | M |
| 16 | LTSRC | Litter source | LT | cf. LTSRC |  |
| 17 | TYPPL | Type of polymer | LT | cf. TYPPL |  |
| 18 | LTPRP | Litter properties (including polymer shapes and colours) | LT | cf. LTPRP multiple flags possible (separate multiple entries with “~” (ascii 126)) |  |
| 19 | AMLNK | Analytical method link (unique for file) | LT | 1-999 |  |
| 20 | RSRVD | Reserved | LT |  |  |
| 21 | DCFLG | Data centre flag - Reserved | LT | multiple options possible |  |

## RECID 38	Biological Community Abundance/Biomass Record

| Biological Community Abundance/Biomass Record – mandatory record for PP, ZP, PB, ZB – |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Data Type |  |  |  |  | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | PP | PB | ZP | ZB |  | ‘38’ | M |
| 2 | CRUIS | Cruise identifier (series of sampling occasions) (must be unique for file) | PP | PB | ZP | ZB |  | Any character 0–9, A–Z etc. | M |
| 3 | STNNO | Station identification /Sampling event ID (must be unique for CRUIS) | PP | PB | ZP | ZB |  | Any character 0–9, A–Z etc. | M |
| 4 | TRANS | Transect ID |  | PB |  | ZB |  |  | M if transect data |
| 5 | SMPNO | Sample identification (for each sediment core, each sediment grab, each water bottle, each transect section or point) | PP | PB | ZP | ZB |  | Any character 0–9, A–Z etc. For transects, sequential numbering from transect start beginning with 1 | M |
| 6 | MNDEP | Minimum depth of sample (m) For zoobenthos, upper depth of the sediment sample. | PP | PB | ZP | ZB |  | 0–9 | rPlankton & PB (mPP HELCOM) |
| 7 | MXDEP | Maximum depth of sample (m) For zoobenthos, lower depth of the sediment sample. | PP | PB | ZP | ZB |  | 0–9 | rPlankton & PB (mPP HELCOM) |
| 8 | SPECI | Latin name of species (or aggregated genus/family) | PP | PB | ZP | ZB |  | cf. WoRMS, cf. PEG biovolume | M |
| 9 | RLIST | Reference code list used for species ID | PP | PB | ZP | ZB |  | cf. RLIST | M |
| 10 | SFLAG | Species flag | PP | PB | ZP | ZB |  | cf. SFLAG multiple options possible (separate multiple entries with “~” (ascii 126)) |  |
| 11 | STRID | Stratum ID |  | PB |  |  |  | cf. STRID | mPB if stratum sampling is used |
| 12 | SIZCL | Size class (value from reference in SIZRF) | PP |  | ZP |  |  | 0–9 | rPP |
| 13 | RSRVD | Reserved |  |  |  |  |  |  |  |
| 14 | MAGNI | Total Magnification | PP |  | ZP |  |  | 0–9 | rPlankton |
| 15 | COEFF | Coefficient | PP |  |  |  |  | 0–9 | rPP |
| 16 | TRPHY | Trophic status of the species | PP |  |  |  |  | cf. TRPHY |  |
| 17 | STAGE | Developmental stage of specimens | PP |  | ZP | ZB |  | cf. STAGE | rZP |
| 18 | SEXCO | Sex code |  |  | ZP | ZB |  | cf. SEXCO |  |
| 19 | PARAM | Biological community parameter | PP | PB | ZP | ZB |  | cf. parameters listed in Pargroup B-COM | M |
| 20 | MUNIT | Measurement unit | PP | PB | ZP | ZB |  | cf. MUNIT | M |
| 21 | VFLAG | Validity flag | PP | PB | ZP | ZB |  | cf. VFLAG multiple flags possible (separate multiple entries with “~” (ascii 126)) | R |
| 22 | QFLAG | Qualifier flag | PP | PB | ZP | ZB |  | cf. QFLAG multiple flags possible (separate multiple entries with “~” (ascii 126)) | O |
| 23 | VALUE | Value measured | PP | PB | ZP | ZB |  | Any format | M |
| 24 | CPORT | Number of portions counted from split sample | PP |  | ZP |  |  | 1–9999 | rZP |
| 25 | SDVOL | Sedimentation volume (ml) | PP |  |  |  |  | 0–9 | rPP |
| 26 | AMLNK | Analytical method link (unique for file) | PP | PB | ZP | ZB |  | 1–999 |  |
| 27 | DCFLG | Data centre flag - Reserved | PP | PB | ZP | ZB |  | multiple options possible |  |

# Method RecordS

## RECID 20	Sampling Method Record

| Sampling method record – mandatory record for CF, CS, PP, ZP, PB, ZB, LT – – optional record for CW – |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Data Type |  |  |  |  |  |  |  | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | CF | CW | CS | PP | PB | ZP | ZB | LT | ‘20’ | M |
| 2 | SLABO | Sampling laboratory code | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. RLABO | M |
| 3 | SMLNK | Sampling method link (unique for entire file) | CF | CW | CS | PP | PB | ZP | ZB | LT | 1–999 | M |
|  |  | Sampler information |  |  |  |  |  |  |  |  |  |  |
| 4 | SMTYP | Sampler type | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. SMTYP | rPP mPB rCS rZB mPassive sampling |
| 5 | NETOP | Net opening width (m) | CF |  |  |  |  |  |  | LT | 0–9 |  |
| 6 | MESHS | Mesh size of net or sieve (µm) | CF |  |  | PP | PB | ZP | ZB | LT | 0–9 | rZ |
| 7 | SAREA | Sampler area (cm2) (includes field of view and swept area for some data types. See definitions) |  | CW | CS |  | PB | ZP | ZB | LT | 0–9 | rZP/ZB, mPB mPassive sampling, mLT |
| 8 | LNSMB | Length of sampler (core) barrel (cm) |  |  | CS |  |  |  | ZB | LT | 0–9 |  |
| 9 | SPEED | Speed (ex. trawls) (knots) | CF |  |  | PP |  |  |  | LT | 0–9 |  |
| 10 | PDMET | Plankton (or eutrophication) sampling depth method |  | CW |  | PP |  | ZP |  |  | cf. PDMET | rPlankton |
| 11 | SPLIT | Sample splitting technique |  |  |  |  |  | ZP |  |  | cf. SPLIT |  |
| 12 | OBSHT | Observation height (from surface) (metre) |  |  |  |  |  |  |  | LT |  |  |
| 13 | DURAT | Duration of haul or sampling (minutes) | CF |  |  |  |  |  |  | LT |  |  |
| 14 | DUREX | Duration of exposure in days |  | CW | CS |  |  |  |  |  |  | mPassive sampling |
| 15 | ESTFR | Estimated water sampling rate (flow) in litres per day |  | CW | CS |  |  |  |  |  |  | mPassive sampling |
| 16 | DCFLG | Data centre flag - Reserved | CF | CW | CS | PP | PB | ZP | ZB | LT | multiple options possible |  |

## RECID 21	Analytical Method Record

| Analytical Method Record –mandatory for certain parameters– |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name |  | Data type |  |  |  |  |  |  |  |  | Valid value | Mandatory/ Recommended/ Optional |
| 1 | RECID |  | Record identifier |  | CF | CW | CS | PP | PB | ZP | ZB | LT | ‘21’ | M |  |
| 2 | AMLNK |  | Analytical methods link (unique for entire file) |  | CF | CW | CS | PP | PB | ZP | ZB | LT | 01–999 sequential numbering | M |  |
| 3 | ALABO |  | Analytical laboratory code |  | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. RLABO | M |  |
| 4 | METDC |  | Method documentation |  | CF | CW | CS | PP | PB | ZP | ZB | LT | User defined code to match method document submitted to ICES |  |  |
| 5 | REFSK |  | Reference source or key |  | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. REFSK | r Fish disease r Biological effects |  |
| 6 | METST |  | Method of storage |  | CF | CW | CS | PP | PB | ZP | ZB |  | cf. METST |  |  |
| 7 | METFP |  | Method of chemical fixation/preservation |  | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. METFP |  |  |
| 8 | METPT |  | Method of pretreatment |  | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. METPT multiple flags possible (separate multiple entries with “~” (ascii 126)) |  |  |
| 9 | METCX |  | Method of chemical extraction |  | CF | CW | CS |  |  |  |  | LT | cf. METCX multiple flags possible (separate multiple entries with “~” (ascii 126)) | rCS |  |
| 10 | METPS |  | Method of purification/separation |  | CF | CW | CS |  |  |  |  | LT | cf. METPS |  |  |
| 11 | METOA |  | Method of analysis |  | CF | CW | CS | PP | PB | ZP | ZB | LT | cf. METOA | R |  |
| 12 | AGDET |  | Age determination |  | CF |  |  |  |  |  |  |  | cf. AGDET | R |  |
|  |  |  | Bioassays |  |  |  |  |  |  |  |  |  |  |  |  |
| 13 | SREFW |  | Source of reference seawater |  |  | CW | CS |  |  |  |  |  | cf. SRCWT | rEmbryo Assays |  |
|  |  |  | Live test organism used in bioassay |  |  |  |  |  |  |  |  |  |  |  |  |
| 14 | SPECI |  | In vivo/In vitro test organism or cell line |  |  | CW | CS |  |  |  |  |  | cf. VIVIT | rGeneral Quality Assessment Assays |  |
| 15 | RLIST |  | Reference code list used for species ID |  |  | CW | CS |  |  |  |  |  | cf. RLIST |  |  |
| 16 | ORGSP |  | Origin of test specimen |  |  | CW | CS |  |  |  |  |  | cf. ORGSP | rGeneral Quality Assessment Assays |  |
|  |  |  | Information for conversions |  |  |  |  |  |  |  |  |  |  |  |  |
| 17 | SIZRF |  | Size class reference list |  |  |  |  | PP |  | ZP |  |  | cf. SIZRF | rPP |  |
| 18 | FORML |  | Formula used in calculation |  | CF | CW | CS | PP | PB | ZP | ZB |  | cf. FORML |  |  |
|  |  |  | Information on accreditation |  |  |  |  |  |  |  |  |  |  |  |  |
| 19 | ACCRD |  | Accredited laboratory for the linked parameter |  | CF | CW | CS | PP | PB | ZP | ZB |  | ’Y’ for Yes or ‘N’ for No |  |  |
| 20 | ACORG |  | Accrediting organisation |  | CF | CW | CS | PP | PB | ZP | ZB |  | cf. ACORG |  |  |
| 21 | DCFLG |  | Data centre flag - Reserved |  | CF | CW | CS | PP | PB | ZP | ZB | LT | multiple options possible |  |  |

# Quality Assurance (QA) Records

## RECID 93	Reference Material Record

| Quality Assurance – Reference Material – optional record for ICES database / mandatory for certain assessments and Commissions – for example, this record is mandatory for OSPAR for some parameters |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field Name | Valid Value | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record ID | ‘93’ | M |
| 2 | AMLNK | Analytical methods link – unique number. ALABO information from 21 record, PARAM and MATRX from 10 record | 1–999 | M |
| 3 | QALNK | QA information link | 1–99 | M |
| 4 | CONCH | Type of reference material | cf. CONCH | M |
| 5 | CRMCO | Reference material code | cf. CRMCO | M |
| 6 | CRMMB | Reference material – basis of determination used in control chart analysis Note:The same basis should be used as that of the reference material to allow comparison. | cf. BASIS |  |
| 7 | CRMMV | Reference material mean value found – value | 0–9 Any format |  |
| 8 | MUNIT | Measurement unit | cf. MUNIT |  |
| 9 | CRMSD | Reference material’s standard deviation - standard deviation | 0–9 Any format |  |
| 10 | CRMNM | Control chart – number of measurements | 0–9 |  |
| 11 | CRMPE | Control chart – period in weeks | 0–9 |  |
| 12 | DCFLG | Data centre flag - Reserved | multiple options possible |  |

## RECID 94	Intercomparison Record

| Quality Assurance – Intercomparison Participation Quality Assurance – optional record for ICES database / mandatory for certain assessments – |  |  |  |  |
| --- | --- | --- | --- | --- |
| No | Field Code Foreign key in bold, primary key in italics | Field name | Valid values | Mandatory/ Recommended/ Optional |
| 1 | RECID | Record identifier | ‘94’ | M |
| 2 | AMLNK | Analytical methods link | 1–999 sequential numbering according to ALABO | M |
| 3 | ICLNK | Intercomparison link | 1–99 | M |
| 4 | ICCOD | Intercomparison exercise code | cf. ICCOD | M |
| 5 | ICLAB | Intercomparison: lab participation code – can be ALABO | cf. RLABO or any character 0–9, A–Z etc. |  |
| 6 | DCFLG | Data centre flag - Reserved | multiple options possible |  |

# SUPPORTING INFORMATION

Example 1	Horizontal Transect

Example 2	Vertical Transect

# CHANGE LOG

ERF 3.2 was released for reporting data on 7 Jan. 2005. Change log started 2005-07-29

| Edition date | Revision | Reason for revision |
| --- | --- | --- |
| Release date 2005-01-07 | Version 3.2.1 |  |
| 2005-07-29 | Expand SMPNO field to 20 |  |
| 2005-08-10 | Intro to sample records |  |
| 2005-09-05 | Multiple options possible (separate multiple entries with “~” (ascii 126)) for POSYS |  |
| 2005-09-27 | Multiple options removed from SIZGL & SIZGW |  |
| 2005-11-09 | “M” removed from CRMMB and note added to inform of ICCOD code changes. DATSU texts edited to clarify “mandatory” for OSPAR. Sizes of fields expanded. Final values will be described in DATSU upon completion. |  |
| 2006-01-10 | Added purpose of monitoring field “PURPM” in record 91. Changed version number to 3.2.1 |  |
| 2006-02-09 | Allow CW for PDMET and DEPHL |  |
| 2006-03-29 | Char is allowed for STNNO and definition changed from “number” to “ID” |  |
| 2006-05-23 | Clarified text for sediment grab SUBNO’s |  |
| Release date 2006-07-18 | Changed version number to 3.2.2 Record 38: - Deleted SIZGL(sizegroup length) field in record 38 - Deleted SIZGW (sizegroup width) field in record 38 - Added SIZCL (sizeclass) field in record 38. NOTE that the number of fields remain the same in record 38. SIZCL replaces the SIZGL field and checks will be added to DATSU to ensure correct usage between format versions. Record 21: - Added SIZRF (sizeclass reference list) in record 21. - Added FORML (formula) field in record 21. | HELCOM PEG stated that fields are insufficient for reporting size classes of phytoplankton. New fields needed to meet PEG’s requirements. |
| 2006-07-19 | - Web link to FORML - SIZCL changed to integer - RSRVD added to “blank” field in record 38 |  |
| 2006-08-01 | MSTAT can have multiple entries | Stations can have more than one point source impact |
| 2006-08-31 | Removed italics from MNDEP/MXDEP | Italics are misleading - interpreted as a key |
| 2007-03-01 | - In SMVOL, volume information changed from (litre) to specifying litre for PP, cubic meter for ZP and litre sediment for ZB. FLVOL definition expanded to : “Calculated volume of water (cubic meter)” |  |
| 2007-03-28 | - METCX can have multiple entries so allow “~” | Reduce amount of requests for chemical combinations in new RECO METCX codes. |
| 2007-06-01 | - MESHS and SAREA in sampling method are now allowed for datatype PB | Data submitter has data to report. |
| 2007-08-01 | - MUNIT reference to RECO was added in record 93 |  |
| Release date 2008-02-27 2007-09-04 2008-01-17 2008-02-15 | Changed version number to 3.2.3 - PRFLG depth/pressure flag added - STNNO was changed to TRANS ID to link to the 92 record. FNFLA made mandatory for PB Added Data Centre flag field Clarified NODIS description | - Pressure needed for 3.2 data entering the Hydrochem database. - Records 31, 32, 33 are now checked by DATSU. This required a field change. Samples without fauna can occur for PB. ESD will not allow entry unless FNFLA is filled Need to flag legacy data where errors or omissions in older data would otherwise prevent entry to ESD. Data submitters do not understand this field which is a requirement for assessing fish survey data by WGPDMO. |
| 2008-04-10 | WIRAN field opened for PP, and removed for ZB | Error corrected |
| 2008-07-24 | Added reason for changes to change log table | For clarity |
| 2009-04-27 | Added PP to STAGE in 38 record | Omission corrected |
| 2010-05-28 | - Added PP to NPORT in 34 record and added “split” to definition - Added PP to CPORT in 38 record and added “split” to definition | Clarified usage. For split samples only, i.e., splitting one bottle. NPORT/CPORT unlikely for PP but could happen. Benthos more unlikely and field remains not active. |
| 2010-06-14 | Enlarged SMLNK field from 2 digits to 3 digits. | Fish disease data need speed reported for each haul and they have more than 99 hauls per year. |
| 2010-06-25 | SMVOL – added rPlankton | Omission for PP corrected. |
| Release date 2012-11-15 2010-2011 | Changed version number to 3.2.4 Quality Assurance: - Added QA accreditation field to record 21 - Added QA accreditation organisation to record 21 - UNCRT/METCU fields now mandatory for chemicals | HELCOM MONAS 2012 approved MCWG, HELCOM MONAS, OSPAR MIME |
| 2011-03-09 | Support transect points/section information - Deletion of 31, 32 and 33 record - New record – transect record 40 - Deletion of fields in 34 record: MXVEG, SPVEG, RLIST – no data in database and order not changed - Addition of fields for 34 record: PRSUB, TRSCS, TRSCE, TRCSD, TRCED - In 92 record, field TRANS replaced with RSRVD so it can no longer be used - ZB allowed as transect data | WKMDB specifications for phytobenthos transect data. |
|  | - SAREA “Sampler area opening” changed to “Sampler area including field of view” | WKMDB: The definition was confusing. |
| 2011-07-21 | Description change for coordinates - WGS84 recommended - Decimal degrees now allowed | Request from OSPAR |
| 2012-03-08 | Revise definition of R21:SPECI and link to RECO table VIVIT (In vivo/in vitro test) | SGIMC2011/WGBEC2012 expansion of field to include in vitro tests (Cell lines) |
| 2013-03-01 | Edited 3.2.4 links to new web site locations |  |
| 2013-06-21 | Added DTYPE ZB to R38:TRANS | Omission corrected |
| 2013-06-24 | Added meters to definitions of PRFLG, TRCSD and TRCED | Clarification |
| 2013-07-17 | Removed rOSPAR on CRMMB and CRMMV | OSPAR no longer requires these – now requires UNCRT |
| 2014-02-19 | Correction of text: From ‘Yes’ to ‘Y’ for Yes and From ‘No’ to ‘N’ for No | Clarification |
| 2014-05-21 | Correction of text: LMQNT is only mandatory for some programmes and some chemicals – changed to R as this is controlled in DATSU | Clarification |
| 2014-12-12 | Correction to CRMPE – added “weeks” | Clarification |
| 2015-01-20 | R94:ICCOD made Mandatory | Originally a system was planned where 94:ICLAB could link to another database and get all ICCOD participation results for the lab reported in ICLAB. ICCOD was therefore not necessary if ICLAB was filled. This will not be done so ICCOD is now mandatory if the 94 record is reported. Record 94 is still optional. |
| Release date 2015-03-30 | Changed version number to 3.2.5 Litter record 36 SAREA in 20 record opened for CW New fields added in 20 record: Observation height (OBSHT), Duration of haul (DURAT) New field in 91 record: EDATE | Allow reporting of litter in all data types and as a separate data type New fields needed for passive sampling data in seawater (CW). |
|  | LTPRP multiples allowed | Descriptions can be colour/shape/size etc. |
| 2015-08-01 | Diagrams edited: litter in biota to be reported in 10 record with PARAM starting with “LT%” | Clarify difference between reporting litter (ex. microplastics) found inside organism and litter gathered in same net (macrolitter) |
| 2015-08-11 | Fields DUREX and ESTFR added to the 20 record sampling method record | Needed to support MCWG2011 mandatory requirements for passive sampling |
| 2015-10-19 | Allowed METPT for all data types | Labs report the field and errors are first found when data are prevented upload in DOME |
| 2015-11-11 | Field ESTSR changed to ESTFR | Conflict in database with legacy code ESTSR |
| 2015-11-20 | Opened SEXCO for zoobenthos data | Request from submitter for easier mapping |
| 2016-01-21 | Added field QFLAG to record 36 for litter data | Request from submitter who says weights are often less than scale’s capability |
|  | Clarified SAREA. | This field is data type dependent but these definitions are not found here. See FAQ document. |
| 2016-02-10 | Corrected RECO link for PURPM | Was corrupted |
| 2016-02-25 | Added mCW for DEPHU/DEPHL | Clarified that Depth is Mandatory for most parameters. Exceptions are effects parameters PNR and MORT% |
| 2016-03-17 | Changed MAGNI description to Total Magnification | Total magnification – ocular, objective, plus base. Requested by HELCOM indicator producers |
|  | MNDEP and MXDEP mandatory for HELCOM | Requested by HELCOM indicator producers |
| 2016-04-30 | DURAT expanded to Duration of haul or sampling | Field can now be used for Beach litter |
| 2017-10-01 | RECO link added to STATN | The request program has been expanded to include stations and bulk uploads of stations. This required stations to be in RECO, the ICES vocabulary database. |
| 2018-04-26 | SAREA definition extended to include “swept area” for litter trawls and is mandatroy for MUNIT */haul | WGML compromise for omitting raw data for calculations. (Raw data would have distance and end transect trawl coordinates) |
| 2018-06-28 | Added hyperlink to “Field description” | Submitters don’t remember to use selRep so link was added |
| 2018-09-05 | Clarified STNNO - Station identification /Sampling event ID (must be unique for CRUIS) | Submitters have used the same station number with different dates for the same cruise when returning to a station. Extraction of data goes wrong. The station must be renumbered. For example Mystation_1, Mystation_2 |
| 2019-02-21 | QFLAG no longer allows multiples for data with MYEAR >= 2018 | Reporting of less thans has been inconsistent and affects the calculations by the automatic assessment tool. See FAQ for more information. |
| 2019-08-07 | Text added: - in Record 40, field TRANS: added “TRANS must be unique for STNNO” - in all ‘DCFLG’ fields for all record types, added “multiple options possible” - added Column ‘No’ with reference to the fields’ order number within the record. | Text edits for clarification |
| 2019-09-17 | Litter sample record information edited to match the structure diagram and present database options | Contextual edit to align with present format for litter data |